# Design of an Intelligent Analog Front-End for EV Battery Diagnostics, Adaptive Calibration, Thermal Runaway Prevention, Aging-Aware Protection, and Fast-Fault Safety Monitoring

#### Project report submitted to

Indian Institute of Information Technology, Nagpur,

in partial fulfillment of the requirements for the Final Year Project

at

# DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

#### by

| Palash Goswami (BT22ECE073) |  | Chaahat Singal (BT22ECE099) |  |
| --- | --- | --- | --- |

Under the supervision of

Dr. Shankar Bhattacharjee

Dr. Girish Ghivela

Session Period: Jan to May 2026

## भारतीय सूचना प्रौद्योगिकी संस्थान, नागपुर

## INDIAN INSTITUTE OF INFORMATION TECHNOLOGY, NAGPUR

(An Institution of National Importance by Act of Parliament)

Survey No. 140, 141/1, Behind Br. Sheshrao Wankhede Shetkari Sahakari

SootGirni, Nagpur,441108.

# Design of an Intelligent Analog Front-End for EV Battery Diagnostics, Adaptive Calibration, Thermal Runaway Prevention, Aging-Aware Protection, and Fast-Fault Safety Monitoring

#### Project report submitted to

Indian Institute of Information Technology, Nagpur,

in partial fulfillment of the requirements for the Final Year Project

at

# DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

#### by

| Palash Goswami (BT22ECE073) |  | Chaahat Singal (BT22ECE099) |  |
| --- | --- | --- | --- |

Under the supervision of

Dr. Shankar Bhattacharjee

Dr. Girish Ghivela

Session Period: Jan to May 2026

## भारतीय सूचना प्रौद्योगिकी संस्थान, नागपुर

## INDIAN INSTITUTE OF INFORMATION TECHNOLOGY, NAGPUR

(An Institution of National Importance by Act of Parliament)

Survey No. 140, 141/1, Behind Br. Sheshrao Wankhede Shetkari Sahakari

SootGirni, Nagpur,441108.

## Department of Electronics and Communication Engineering

Indian Institute of Information Technology, Nagpur

## Declaration

We, Chaahat Singal and Palash Goswami hereby declare that this project work titled “Design of an Intelligent Analog Front-End for EV Battery Diagnostics, Adaptive Calibration, Thermal Runaway Prevention, Aging-Aware Protection, and Fast-Fault Safety Monitoring” is carried out by me/us in the Department of Electronics and Communication Engineering of Indian Institute of Information Technology, Nagpur. The work is original and has not been submitted earlier whole or in part for the award of any degree/diploma at this or any other Institution /University.

Date:

| Sr. No | Name | Enrollment No | Signature |
| --- | --- | --- | --- |
| 1. | Chaahat Singal | BT22ECE099 |  |
| 2. | Palash Goswami | BT22ECE073 |  |

## Declaration

We, Chaahat Singal, Palash Goswami Enrollment No (BT22ECE099, BT22ECE073), understand that plagiarism is defined as any one or the combination of the following:

- Uncredited verbatim copying of individual sentences, paragraphs or illustrations (such as graphs, diagrams, etc.) from any source, published or unpublished, including the internet.

- Uncredited improper paraphrasing of pages or paragraphs (changing a few words or phrases, or rearranging the original sentence order).

- Credited verbatim copying of a major portion of a paper (or thesis chapter) without clear delineation of who did or wrote what. (Source: IEEE, the institute, Dec.2004) I have made sure that all the ideas, expressions, graphs, diagrams, etc. that are not a result of my own work, are properly credited. Long phrases or sentences that had to be used verbatim from published literature have been clearly identified using quotation marks.

I affirm that no portion of my work can be considered as plagiarism and I take full responsibility if such a complaint occurs. I understand fully well that the guide of the thesis may not be in a position to check for the possibility of such incidences of plagiarism in this body of work.

Date:

| Sr. No | Name | Enrollment No | Signature |
| --- | --- | --- | --- |
| 1. | Chaahat Singal | BT22ECE099 |  |
| 2. | Palash Goswami | BT22ECE073 |  |

ELECTRONICS AND COMMUNICATION ENGINEERING IIIT, NAGPUR

Certificate

This is to certify that the project titled “Design of an Intelligent Analog Front-End for EV Battery Diagnostics, Adaptive Calibration, Thermal Runaway Prevention, Aging-Aware Protection, and Fast-Fault Safety Monitoring”, submitted by Palash Goswami, Chaahat Singal in partial fulfilment of the requirements for the award of the degree of Bachelor of Technology in Electronics and Communication Engineering, IIIT Nagpur. The work is comprehensive, complete and fit for final evaluation.

Date: 15th May, 2026

Dr. Shankar Bhattacharjee	            Dr. Girish Ghivela

Assistant Professor, ECE, 	              Assistant Professor, ECE,

IIIT, Nagpur 	                IIIT, Nagpur

Industry Expert’s Signature  	               Academic Expert’s Signature

Dr. Harsh Goud

HOD, Electronics & Communication Engineering

IIIT, Nagpur

# Acknowledgement

It gives us immense pleasure to present this final year project report, a culmination of dedicated effort, learning, and continuous exploration. This work would not have been possible without the guidance, encouragement, and support of several individuals and institutions, to whom we owe our heartfelt gratitude.

We express our sincere appreciation to Dr. Shankar Bhattacharjee and Dr. Girish Ghivela, our project supervisor, for his exceptional mentorship, constructive feedback, and invaluable insights throughout the project. His constant support and academic guidance have been instrumental in shaping the direction and quality of this work.

We extend our respectful thanks to Dr. Harsh Goud, Head of the Department, Electronics and Communication Engineering, Indian Institute of Information Technology, Nagpur, for providing us with the necessary academic environment, departmental facilities, and motivation that greatly facilitated the completion of this project.

We also acknowledge the Department of Electronics and Communication Engineering, IIIT Nagpur, for providing access to laboratory infrastructure, computational resources, and administrative assistance essential for carrying out the research. We are equally grateful to all faculty members, lab staff, and peers who supported us through their valuable suggestions and cooperation.

Finally, we express our profound gratitude to our families and friends whose unwavering support, encouragement, and understanding sustained us throughout this academic journey. Their motivation has been a constant source of strength.

Abstract

Electric vehicle battery packs require safe, accurate, and fast diagnostic systems because battery failure can occur through multiple mechanisms: over-current stress, sensor offset, thermal rise, internal degradation, poor cooling, and thermal runaway. Conventional battery management systems often depend on fixed temperature, voltage, or current thresholds. Such systems are useful for basic protection, but they usually respond late to internal thermal acceleration, do not dynamically adapt to aging, and may not distinguish between slow thermal hazards and fast electrical faults. This project presents a simulation-validated intelligent analog front-end for EV battery diagnostics and protection that combines calibrated sensing, electro-thermal modeling, aging-aware current governance, ADC/SAR support, RLS-based internal resistance estimation, and hierarchical fast-fault shutdown logic.

The proposed architecture is implemented mainly in LTspice using a first-order battery equivalent circuit model, coupled core-surface thermal network, behavioral heat-generation equations, sensor calibration paths, RC-qualified protection logic, and sampled ADC/SAR blocks. Python is used for post-simulation analysis, multi-case comparison, policy validation, and RLS-based estimation. The system is validated under six representative scenarios: baseline, thermal derating, runaway prevention, aging warning, severe aging, and fast fault. Results show that the controller provides graded thermal derating, early runaway precursor detection, aging-sensitive current scaling, and sub-10 ms fault protection. The project demonstrates that an EV battery front-end can be designed not only as a measurement circuit but also as a predictive safety and diagnostic layer suitable for future ASIC or embedded BMS integration.

Table of Contents

Abstract	7

List of Figures	11
List of Tables	12
List of Symbols, Abbreviations and Nomenclature	13
1. Introduction	14

1.1 Background	14

1.2 Need for Intelligent Battery Protection	14

1.3 Motivation	15

1.4 Project Contribution	15

2. Literature Review	16

2.1 Conventional Battery Management Systems	16

2.2 Thermal Runaway and Derivative-Based Warning	16

2.3 Internal Resistance and Aging	16

2.4 Calibration and ADC in Battery Front-Ends	16

2.5 Research Gap Addressed                                                                                            16

3. Objectives and System Requirements	18

3.1 Overall Objective	18

3.2 Specific Objectives	18

3.3 Engineering Requirements	19

3.4 Design Philosophy	19

4. Proposed System Architecture	20

4.1 Functional Blocks	20

4.2 High-Level Signal Flow	20

4.3 Why This Architecture Is Different	21

5. LTspice Circuit Implementation	22

5.1 Battery Electrical Model	22

5.2 Electro-Thermal Heat Model	22

5.3 Protection Logic Implementation	22

5.4 Measurement and Sensor Trust	23

6. Adaptive Calibration Subsystem	24

6.1 Objective	24

6.2 Problem Solved	24

6.3 Circuit Approach	24

6.4 Quantitative Result	25

6.5 Significance	26

7. ADC and SAR Monitoring Subsystem	27

7.1 Objective	27

7.2 Sample-and-Hold and Quantization	27

7.3 SAR-Like Voltage Conversion	27

7.4 ADC Result Interpretation	28

7.5 Significance	28

8. RLS-Based Internal Resistance Estimation	29

8.1 Objective	29

8.2 Why RLS Was Used	29

8.3 Connection with Aging-Aware Protection	30

8.4 Significance	30

9. Electro-Thermal Protection and Fast Derating	31

9.1 Objective	31

9.2 Thermal Stress and Current Scaling	31

9.3 Case 2: Thermal Derating	31

9.4 Quantitative Result	32

9.5 Significance	33

10. Aging-Aware Policy and Health Scoring	34

10.1 Objective	34

10.2 R0eff and Age Stress	34

10.3 Case 4 vs Case 5	34

10.4 Significance	36

11. Fast Fault Protection Layer	37

11.1 Objective	37

11.2 Circuit Approach	37

11.3 Case 6 Result	37

11.4 Significance	38

12. Multi-Case Validation and Results	39

12.1 Validation Philosophy	39

12.2 Case Definitions	40

12.3 Case 3: Runaway Prevention	41

12.4 Comparative Result Dashboard	42

12.5 What Each Feature Solves	43

13. Engineering Significance and Future Scope	44

13.1 Engineering Significance	44

13.2 Comparison with Simple Threshold Protection	44

13.3 Limitations	44

13.4 Future Scope	45

14. Conclusion	46

References	47

Appendix A: Detailed Feature-to-Objective Mapping	48

Appendix B: Algorithmic Flow Used in the Protection Stack	49

B.1 Calibration Algorithm	49

B.2 Thermal Derating Algorithm	49

B.3 Aging Policy Algorithm	49

B.4 Fast Fault Algorithm	50

Appendix C: Case-by-Case Interpretation for Viva	51

C.1 Case 1 - Baseline	51

C.2 Case 2 - Thermal Derating	51

C.3 Case 3 - Runaway Prevention	51

C.4 Case 4 and Case 5 - Aging Policy	51

C.5 Case 6 - Fast Fault	51

Appendix D: Key LTspice Behavioral Blocks	54

D.1 Representative Equations	55

Appendix E: Selected Figures for Final Demonstration	56

List of Figures

Figure 4.1: Integrated LTspice top-level architecture of the proposed EV battery diagnostic and protection front-end.	21

Figure 4.2: ADC/SAR and sampled-interface region of the LTspice implementation.	21

Figure 6.1: Calibration bench trajectory showing REF/MEAS switching and 
corrected output response.	25

Figure 6.2: Calibration error reduction comparing raw and settled-corrected measurement error.	25

Figure 7.1: Current ADC transfer behavior, input-code relation, and sample-wise quantization error.	28

Figure 8.1: Online R0 estimation using Recursive Least Squares.	29

Figure 8.2: Measured voltage compared with RLS-predicted voltage.	30

Figure 9.1: Case 2 thermal derating showing current reduction, dT/dt moderation
, and no-derate comparison.	32

Figure 10.1: Aging-aware policy comparison between Case 4 and Case 5.	34

Figure 10.2: Observed versus predicted current policy using health and resistance features.	34

Figure 11.1: Case 6 fast-fault latency compared with the 10 ms design requirement.	37

Figure 12.1: Case-wise summary table generated from Python post-processing.	38

Figure 12.2: Event timeline table showing key protection and fault events.	39

Figure 12.3: Case 3 runaway-prevention timeline and no-shutdown proxy comparison.	40

Figure 12.4: Comparative dashboard of protection windows, derating benefit, 
current policy, and fast-fault latency.	41

Figure C.1: Detailed thermal derating waveform used for Case 2 defense.	52

Figure C.2: Detailed runaway prevention waveform used for Case 3 defense.	53

Figure E.1: Calibration timing and measurement/reference switching.	56

Figure E.2: Calibration error reduction summary.	56

Figure E.3: Aging policy model validation.	57

Figure E.4: RLS-based online internal resistance estimate.	57

List of Tables

Table 3.1: Engineering requirements and project implementation.	19

Table 6.1: Calibration performance metrics.	25

Table 9.1: Thermal derating quantitative results.	32

Table 10.1: Aging-aware policy comparison for Case 4 and Case 5.	34

Table 11.1: Fast-fault timing and latency results.	37

Table 12.1: Validation case definitions.	39

Table 12.2: Runaway-prevention phase durations.	40

Table 12.3: Feature-to-objective validation summary.	42

Table 13.1: Comparison between conventional threshold BMS and proposed architecture.	43

Table A.1: Detailed feature-to-objective mapping.	47

Table D.1: Key LTspice behavioral blocks.	54

List of Symbols, Abbreviations and Nomenclature

| Abbreviation | Full Form |
| --- | --- |
| BMS | Battery Management System |
| EV | Electric Vehicle |
| AFE | Analog Front-End |
| ECM | Equivalent Circuit Model |
| OCV | Open Circuit Voltage |
| SOC | State of Charge |
| SOH | State of Health |
| ADC | Analog-to-Digital Converter |
| SAR | Successive Approximation Register |
| DAC | Digital-to-Analog Converter |
| S/H | Sample-and-Hold |
| RLS | Recursive Least Squares |
| RC | Resistor-Capacitor |
| OC | Over-Current |
| UV | Under-Voltage |
| SC | Short-Circuit |
| dT/dt | Rate of change of temperature |
| R0 | Ohmic Internal Resistance |
| R0eff | Effective Internal Resistance |
| ASIC | Application Specific Integrated Circuit |
| FPGA | Field Programmable Gate Array |
| LTspice | Linear Technology SPICE Simulator |

Chapter 3

Introduction

1.1 Background

Electric vehicles depend heavily on lithium-ion battery packs. These packs must deliver high power, accept fast charging, and remain safe across different ambient temperatures, aging conditions, current transients, and fault events. Unlike small portable batteries, EV packs contain many cells connected in series and parallel, so local degradation or abnormal heating can propagate into a system-level hazard. Battery safety therefore cannot rely on a single alarm signal. A realistic protection system must monitor electrical, thermal, and aging-related behavior simultaneously.

The analog front-end is the first layer of a battery management system. It senses voltage, current, and temperature-related signals and sends them to the digital controller. If this front-end contains offset, drift, slow response, or poor fault qualification, the higher-level controller will make decisions based on unreliable information. For this reason, this project focuses on the design of an intelligent analog front-end that combines measurement, calibration, digitization, safety logic, and post-processing validation.

1.2 Need for Intelligent Battery Protection

Traditional BMS designs usually include over-voltage, under-voltage, over-current, and over-temperature cutoffs. These are necessary, but they are not sufficient for advanced EV operation. A thermal runaway event may begin internally before the surface temperature crosses a critical threshold. A degraded cell may appear usable at low current but become unsafe under fast charging. A short-circuit fault requires a millisecond-level response, while thermal derating may occur over hundreds or thousands of seconds. This difference in time scale motivates a hierarchical architecture.

The proposed design addresses this need by implementing multiple protection layers: a fast electrical fault layer, a thermal derating layer, a runaway precursor layer, an aging-aware current policy, and a measurement-trust layer. The result is not only a shutdown circuit but a complete safety stack that can reduce current gradually when possible and force shutdown when required.

1.3 Motivation

• Fast charging increases heat generation and accelerates aging if not controlled.

• Internal resistance increases with degradation and must influence allowable current.

• Sensor offset and drift can cause false decisions; calibration is therefore essential.

• Thermal runaway requires early detection using derivative and stress indicators, not only absolute temperature.

• Fast electrical faults require a separate millisecond-class path, because thermal models are too slow for short-circuit protection.

1.4 Project Contribution

The main contribution of this project is the development of a mixed-signal, simulation-validated EV battery front-end that integrates diagnostic estimation, calibration, ADC support, and multi-timescale protection. The project is intentionally designed as a system architecture rather than a single comparator circuit. Each subsystem has a defined purpose, an implementation method, and a measurable outcome.

• Self-calibrated voltage and current sensing path using reference-mode switching and correction holding.

• Electro-thermal battery model with irreversible, reversible, and reaction heat components.

• Fast derating and shutdown logic using stress fusion and RC filtering.

• Aging-aware current governance based on R0eff, age stress, risk score, and health score.

• ADC/SAR interface for sampled digital monitoring of front-end signals.

• RLS-based internal resistance estimation and measured-vs-predicted voltage validation.

• Case-wise Python validation that converts raw waveforms into engineering metrics.

Chapter 4

Literature Review

2.1 Conventional Battery Management Systems

A conventional BMS performs voltage monitoring, current measurement, temperature sensing, balancing, contactor control, and communication with the vehicle controller. In many simple systems, protective actions are threshold-based: if a voltage, current, or temperature crosses a limit, the system either warns the user or disconnects the pack. This approach is simple and robust, but it has two limitations. First, it is reactive; the system may wait until a severe threshold is reached. Second, it is often non-adaptive; a new cell and aged cell may be treated with similar current limits even though their safe operating regions differ.

2.2 Thermal Runaway and Derivative-Based Warning

Thermal runaway is a positive-feedback process in which heat generation exceeds heat dissipation. Absolute temperature is important, but derivative signals such as dT/dt are also valuable because they indicate acceleration. A cell with a rapidly rising dT/dt can be more dangerous than a cell at a moderately high but stable temperature. Therefore, the proposed design uses Tcell, core-surface thermal gradient, and filtered dT/dt as combined warning variables.

2.3 Internal Resistance and Aging

Internal resistance is an important indicator of battery health. As a cell ages, the effective series resistance increases due to electrochemical degradation, loss of active material, and interface changes. Higher resistance increases I^2R heat generation under the same current. Therefore, a fixed current policy can become unsafe for aged batteries. The proposed architecture computes R0eff and maps it to age stress, health score, and current policy.

2.4 Calibration and ADC in Battery Front-Ends

Battery front-end measurements are affected by input offsets, gain errors, reference drift, ADC quantization, noise, and leakage in sample-and-hold networks. Calibration is used to correct predictable measurement errors. ADC support is required to move from analog simulation toward a digital BMS interface. This project includes reference-mode switching, correction holding, quantized current/temperature ADC blocks, and a SAR-like voltage conversion path.

2.5 Research Gap Addressed

The gap addressed by this project is the lack of a unified student-level design that combines analog front-end calibration, thermal intelligence, aging-aware current scaling, fast electrical fault logic, ADC support, and parameter estimation in one coherent architecture. Instead of demonstrating only one protection condition, the proposed system validates multiple failure modes and shows how each protection layer responds.

Chapter 5

Objectives and System Requirements

3.1 Overall Objective

The objective of this project is to design and validate an intelligent EV battery analog front-end that can sense battery behavior accurately, compensate measurement errors, detect slow and fast hazards, adapt protection based on aging, and provide digitized information for a supervisory controller.

3.2 Specific Objectives

• Develop a first-order equivalent circuit model of a battery cell with current sensing and voltage sensing.

• Build an electro-thermal model that captures core-surface dynamics, ambient cooling, irreversible heat, reversible heat, and reaction heat.

• Design thermal warning, charge limit, and shutdown logic using Tcell, dT/dt, and thermal gradient.

• Implement fast-fault detection using over-current, under-voltage, short-circuit fusion, validation filtering, and hold logic.

• Add adaptive calibration with reference switching and correction output to reduce measurement error.

• Create an ADC/SAR support layer for digitized monitoring of voltage, current, and temperature-related channels.

• Estimate internal resistance using RLS in Python and connect the result with aging-aware protection.

• Validate the full system across six representative operating cases and extract measurable safety windows.

3.3 Engineering Requirements

Table 3.1: Engineering requirements and project implementation.

| Requirement | Reason | Project Implementation |
| --- | --- | --- |
| Stable baseline operation | Avoid nuisance trips under safe operation | Case 1 baseline remains normal without protection action |
| Thermal derating | Reduce stress before shutdown | CHG_LIMIT and ICMD_SCALE_FINAL reduce current |
| Runaway precursor response | Create early warning window | RUNAWAY_PRECURSOR and THERM_SHDN stage response |
| Aging sensitivity | Protect degraded cells | R0eff, AGE_STRESS, HEALTH_SCORE, AGE_SCALE |
| Fast fault reaction | Short-circuit requires ms response | FAST_FAULT_RAW, FAST_TRIP_VALID, FAST_SHDN |
| Measurement reliability | Avoid acting on invalid measurements | REFCHK, SENSOR_FAULT, MEAS_TRUST_OK |
| Digital interface readiness | BMS controller requires sampled data | S/H, quantization, ADC code, SAR logic |

3.4 Design Philosophy

The design philosophy is hierarchical and time-scale aware. Fast electrical faults are handled using a dedicated path with RC validation and hold behavior. Thermal faults are handled using derating and shutdown logic. Aging is handled through a slower health-aware current policy. Calibration and ADC support form the measurement layer beneath these protections. This layered structure makes the system more realistic than a single threshold-based circuit.

Chapter 6

Proposed System Architecture

The proposed system is organized as a mixed-signal safety stack. At the lowest level, the battery equivalent circuit and sensing network generate voltage and current signals. The calibration block corrects offset and drift. The ADC/SAR layer samples and digitizes signals. The electro-thermal model estimates temperature evolution. The protection logic then computes warnings, derating, shutdown, and system state. Python post-processing validates the design using case-wise metrics.

4.1 Functional Blocks

• Battery electrical model: open-circuit source, series resistance, RC polarization branch, shunt current sensing.

• Calibration front-end: reference source, measurement/reference multiplexing, offset correction, filtered output.

• Thermal core-surface model: core heat capacity, surface heat capacity, core-surface resistance, surface-ambient resistance.

• Protection logic: thermal warning, precursor, charge limit, shutdown, aging derate, final shutdown.

• Fast fault logic: over-current, under-voltage, short-circuit fusion, RC validation, latch/hold.

• ADC/SAR interface: sample-and-hold, quantization, bit extraction, SAR comparison, DAC reconstruction.

• Python analytics: case summary, safety windows, policy fitting, calibration improvement, ADC metrics, RLS estimation.

4.2 High-Level Signal Flow

The signal flow begins with the battery current command and cell response. Current passes through a shunt, and voltage is sensed at the battery node. These raw analog values are filtered and passed through the calibration path. Calibrated and sampled values are used by the ADC and by the decision logic. In parallel, the thermal model integrates heat generation to produce Tcell_abs, thermal gradient, and dT/dt. These signals create warning, derating, and shutdown commands. Aging variables alter the allowable current even when temperature is not immediately critical.

Figure 4.1: Integrated LTspice top-level architecture of the proposed EV battery diagnostic and protection front-end.

Figure 4.2: ADC/SAR and sampled-interface region of the LTspice implementation.

4.3 Why This Architecture Is Different

The architecture is not limited to alarm generation. It creates a complete chain: sensing -> calibration -> digitization -> estimation -> policy -> derating -> shutdown. This is important because EV battery protection must be both preventive and reactive. A preventive layer reduces current when stress increases.

Chapter 7

LTspice Circuit Implementation

5.1 Battery Electrical Model

The LTspice implementation uses a first-order battery equivalent circuit model. The model includes a voltage source, ohmic resistance R0, a polarization branch, and a current shunt. This representation is appropriate for front-end diagnostics because it captures both immediate voltage drop due to internal resistance and slower transient behavior due to electrochemical polarization.

V_cell = OCV - I R0 - V_RC

tau = R1 C1

The shunt current is used as the main electrical stress signal. A current command profile drives the system through different operating cases, including baseline operation, thermal stress, runaway stress, aged cell operation, and fast fault conditions.

5.2 Electro-Thermal Heat Model

The thermal section computes heat generation and temperature rise using a core-surface thermal network. The project separates heat into irreversible heat, reversible heat, and reaction heat. This separation is important because Joule heating dominates normal high-current operation, while reaction heat becomes important near runaway conditions.

Q_gen = Q_irr + Q_rev + Q_rxn

Q_irr = I^2 (R0_eff + eta R1_heat)

The thermal model uses a core node and a surface node connected through thermal resistance. This allows the design to monitor both absolute cell temperature and internal thermal gradient. The gradient is used as a precursor feature because internal heat accumulation can precede visible surface temperature rise.

5.3 Protection Logic Implementation

Behavioral sources implement warning, derating, shutdown, and system-state logic. The implementation includes T_WARN, DT_WARN, TABS_WARN, T_CRIT, DT_CRIT, and TABS_CRIT filtered signals. The signals are fused to generate RUNAWAY_PRECURSOR, CHG_LIMIT, THERM_SHDN, SYS_DERATE, SYS_SHDN, and FINAL_SHDN. RC filters are used to avoid switching on numerical noise and to model validation delay.

5.4 Measurement and Sensor Trust

The design contains voltage and current range checks, reference fault checks, and sensor fault logic. MEAS_TRUST_OK gates the final current command scale. This is important because protection decisions should not be based on invalid measurements. If a sensor path becomes unreliable, the system can reduce or block current command execution.

Chapter 8

Adaptive Calibration Subsystem

6.1 Objective

The objective of the calibration subsystem is to reduce measurement error caused by sensor offset and reference drift. In an EV battery front-end, small errors in voltage and current measurement can lead to wrong decisions, especially when estimating internal resistance or detecting under-voltage and over-current conditions.

6.2 Problem Solved

Without calibration, the front-end may confuse offset drift with real battery behavior. This can produce false aging estimates, incorrect derating, or missed protection events. The calibration subsystem solves this by periodically switching the measurement path to a known reference and storing the correction.

6.3 Circuit Approach

The LTspice implementation uses REF_MODE and MEAS_MODE control signals. During measurement mode, the sensed battery voltage passes through the normal path. During reference mode, a known reference voltage is injected into the calibration path. The difference between the observed value and reference is treated as calibration error. The correction is held and used to produce a corrected output.

• V_mux_in switches between sensed battery voltage and calibration reference.

• V_cal_err_raw captures the instantaneous reference error.

• V_cal_hold stores the correction during calibration windows.

• V_cal_out represents the corrected measurement output.

• VCAL_VALID and ICAL_VALID represent calibration validity decisions.

Figure 6.1: Calibration bench trajectory showing REF/MEAS switching and corrected output response.

Figure 6.2: Calibration error reduction comparing raw and settled-corrected measurement error.

6.4 Quantitative Result

Table 6.1: Calibration performance metrics.

| Metric | Value | Meaning |
| --- | --- | --- |
| Raw error RMS | 1.957 V | Uncorrected reference-path error before calibration |
| Settled corrected RMS | 0.419 V | Residual error after correction settles |
| RMS improvement | 78.6% | Reduction in measurement error after calibration |
| Raw max absolute error | 2.629 V | Worst observed uncorrected error |
| Settled max absolute error | 1.301 V | Worst corrected error after settling |

6.5 Significance

The calibration result shows that the project does not simply assume ideal sensing. It models a measurement error source and demonstrates that the analog front-end can reduce error through reference-based correction. This is a key step toward a practical battery diagnostics IC, where offset and drift cannot be ignored.

Chapter 9

ADC and SAR Monitoring Subsystem

7.1 Objective

The objective of the ADC subsystem is to bridge the analog diagnostic front-end with digital battery management logic. A real BMS requires digitized voltage, current, and temperature values. Therefore, this project includes sample-and-hold networks, quantization blocks, binary code generation, and a SAR-like voltage conversion path.

7.2 Sample-and-Hold and Quantization

The sample-and-hold section captures voltage, current, and temperature signals using sampling switches and hold capacitors. Quantization is modeled using behavioral sources that scale the input to an 8-bit code range. The code is then decomposed into individual bit outputs. This validates how analog measurements can be prepared for a digital controller.

Code = floor( limit(V_in / V_FS, 0, 1) * 255 + 0.5 )

7.3 SAR-Like Voltage Conversion

The SAR subsystem uses phased bit decisions and a DAC reconstruction path. During each bit trial, a tentative DAC voltage is formed and compared with the sampled input. The comparator output updates the stored bit value. This subsystem is implemented as a supporting mixed-signal demonstration rather than as a full transistor-level ADC.

Figure 7.1: Current ADC transfer behavior, input-code relation, and sample-wise quantization error.

7.4 ADC Result Interpretation

The ADC plots show that the current channel follows the ideal transfer trend with small quantization and reconstruction error. Temperature ADC tests were also explored, but the final report emphasizes the current and voltage/SAR paths because they are more directly connected to protection decisions. The ADC layer proves that the design can be extended from purely analog protection to sampled supervisory control.

7.5 Significance

Including ADC and SAR support makes the project a mixed-signal front-end rather than only an analog comparator network. This improves the final system story: the analog layer senses and protects, while the digitized signals enable future microcontroller, FPGA, or ASIC implementation.

Chapter 10

RLS-Based Internal Resistance Estimation

8.1 Objective

The objective of the RLS subsystem is to estimate internal resistance online from voltage and current behavior. Internal resistance is a key aging indicator. A degraded battery exhibits larger voltage drop and greater heat generation under the same current. Therefore, estimating R0 helps connect diagnostics with protection policy.

8.2 Why RLS Was Used

A simple pulse method estimates R0 using Delta V / Delta I. This is useful for static tests but becomes less reliable during dynamic current profiles because the voltage response also contains RC polarization and noise. Recursive Least Squares updates parameters over time, allowing the estimator to adapt as new voltage-current samples arrive.

theta_k = theta_{k-1} + K_k ( y_k - phi_k^T theta_{k-1} )

K_k = P_{k-1} phi_k / ( lambda + phi_k^T P_{k-1} phi_k )

Here, theta represents the estimated parameter vector, phi is the regression vector, y is the measured output, P is the covariance matrix, and lambda is the forgetting factor. The estimator was implemented in Python using exported LTspice signals.

Figure 8.1: Online R0 estimation using Recursive Least Squares.

Figure 8.2: Measured voltage compared with RLS-predicted voltage.

8.3 Connection with Aging-Aware Protection

The estimated internal resistance supports the aging-aware policy. As R0 increases, the allowed current should reduce because the same current produces greater heat. In the LTspice design, R0eff is directly used to compute age stress and health score. The Python RLS analysis provides an algorithmic path for estimating this quantity from measured signals in a practical BMS.

8.4 Significance

This block gives the project a diagnostic intelligence layer. The system does not only react to present temperature or current; it can also infer degradation and use that information to modify current policy. This is one of the strongest advanced features of the project.

Chapter 11

Electro-Thermal Protection and Fast Derating

9.1 Objective

The objective of the electro-thermal protection layer is to reduce current before the battery reaches a severe thermal state. A threshold-only design waits for absolute temperature to exceed a limit. The proposed design uses thermal stress, dT/dt, and charge-limit scaling to reduce current earlier.

9.2 Thermal Stress and Current Scaling

Thermal stress is computed as a normalized variable based on cell temperature. Age stress is computed from R0eff. The combined stress is used to produce an advanced current scale. The final current command is selected from charge-limit scale, age scale, and advanced stress scale while also respecting measurement trust.

THERM_STRESS = limit((Tcell_abs - 35) / 20, 0, 1)

RISK_SCORE = limit(0.6 THERM_STRESS + 0.4 AGE_STRESS, 0, 1)

HEALTH_SCORE = limit(1 - RISK_SCORE, 0, 1)

9.3 Case 2: Thermal Derating

Case 2 represents thermal stress with degraded cooling conditions but without immediate runaway. The system detects rising thermal stress and applies current derating rather than forcing immediate shutdown. This is important because EV systems should avoid unnecessary pack disconnection when controlled derating is sufficient.

Figure 9.1: Case 2 thermal derating showing current reduction, dT/dt moderation, and no-derate comparison.

9.4 Quantitative Result

Table 9.1: Thermal derating quantitative results.

| Parameter | Observed Result | Engineering Meaning |
| --- | --- | --- |
| Warning window | 179.14 s | Time available before derating action |
| Current reduction | 17.48% | Reduction in current command after derating |
| I^2 thermal drive reduction | 31.75% | Reduction in Joule-heating driver |
| dT/dt reduction | 67.07% | Strong suppression of thermal acceleration |
| Temperature rise benefit | 64.00% | Lower temperature growth vs no-derate proxy |

9.5 Significance

The thermal derating result proves that the controller does not wait passively for shutdown. It uses an intermediate operating state that lowers stress while maintaining operation. This is closer to real EV safety behavior where power derating is preferred before hard isolation.

Chapter 12

Aging-Aware Policy and Health Scoring

10.1 Objective

The objective of the aging-aware policy is to adapt allowable current based on battery health. A fixed current limit is unsafe because a degraded battery has higher internal resistance and generates more heat under the same current. The design therefore uses R0eff, age stress, risk score, and health score to govern charging current.

10.2 R0eff and Age Stress

R0eff is modeled as a function of base resistance, temperature, SOC, and aging factor. The age stress variable maps R0eff into a normalized degradation score. Severe aging triggers stronger current limitation than mild aging.

R0_eff = R0_base (1 + temperature term + SOC term + aging term)

AGE_STRESS = limit((R0_eff - 0.06) / 0.10, 0, 1)

10.3 Case 4 vs Case 7

Case 4 represents aging warning, while Case 5 represents severe aging. The policy keeps current less restricted in the warning case and significantly reduces current in the severe-aging case. This shows that the controller changes behavior based on battery health rather than using a constant current rule.

Figure 10.1: Aging-aware policy comparison between Case 4 and Case 5.

Figure 10.2: Observed versus predicted current policy using health and resistance features.

Table 10.1: Aging-aware policy comparison for Case 4 and Case 5.

| Metric | Case 4: Aging Warning | Case 5: Severe Aging | Interpretation |
| --- | --- | --- | --- |
| R0eff avg | 0.091 ohm | 0.151 ohm | Higher internal resistance in severe aging |
| Health score min | 0.852 | 0.626 | Health decreases with aging |
| Minimum current scale | 0.700 | 0.251 | Severe aging receives stronger derating |
| Policy model R2 | 0.995 | 0.995 | Current policy is strongly explainable by health/R0/age features |

10.4 Significance

This subsystem solves a real limitation of conventional protection: fixed thresholds ignore degradation. By linking internal resistance and health score to current policy, the architecture becomes adaptive over battery life. This improves safety and also provides a more realistic final-year project story because it connects diagnostic estimation with control action.

Chapter 13

Fast Fault Protection Layer

11.1 Objective

The fast fault layer is designed for events that cannot wait for thermal detection. Short-circuit and over-current faults require a response in milliseconds. Thermal models are too slow for such events, so the project implements a separate electrical fault path.

11.2 Circuit Approach

The fast-fault path uses over-current and under-voltage comparators. Filtered versions of these raw signals are fused to generate short-circuit and fast fault indicators. RC validation rejects narrow spikes, while a hold capacitor keeps the shutdown command active long enough for system response.

• OC_CMP and UV_CMP create analog comparator outputs.

• OC_FAST_RAW and UV_FAST_RAW create digital fault states.

• SC_FAST_RAW detects combined over-current and under-voltage signature.

• FAST_FAULT_RAW is validated by an RC network.

• FAST_TRIP_VALID and FAST_TRIP_HOLD create robust shutdown behavior.

• FINAL_SHDN combines fast electrical shutdown with thermal shutdown.

FINAL_SHDN = FAST_SHDN OR THERM_SHDN_F

11.3 Case 6 Result

Case 6 validates the fast electrical fault response. The observed fault-to-shutdown time is approximately 7.0 ms, which satisfies a 10 ms design target and leaves nearly 3 ms margin.

Figure 11.1: Case 6 fast-fault latency compared with the 10 ms design requirement.

Table 11.1: Fast-fault timing and latency results.

| Signal / Metric | Value | Meaning |
| --- | --- | --- |
| t_fast_fault | 0.005139 s | Fast fault detection time |
| t_fast_shdn | 0.012141 s | Fast shutdown assertion time |
| t_sys_fault | 0.012141 s | System fault assertion time |
| Fault-to-shutdown latency | 7.001 ms | Measured response delay |
| Requirement | 10 ms | Target design limit |
| Margin | 2.998 ms | Timing safety margin |

11.4 Significance

This result proves that the design is multi-timescale. Slow thermal derating protects against gradual heat rise, while fast fault logic handles electrical hazards. Combining both into FINAL_SHDN makes the system more complete and defensible.

Chapter 14

Multi-Case Validation and Results

12.1 Validation Philosophy

The project is validated using multiple cases instead of a single waveform. Each case is designed to test a different subsystem. Case 1 verifies normal behavior. Case 2 tests thermal derating. Case 3 tests runaway prevention. Case 4 tests mild aging. Case 5 tests severe aging. Case 6 tests fast electrical fault response.

Figure 12.1: Case-wise summary table generated from Python post-processing.

Figure 12.2: Event timeline table showing key protection and fault events.

12.2 Case Definitions

Table 12.1: Validation case definitions.

| Case | Scenario | Main Objective | Expected Behavior |
| --- | --- | --- | --- |
| Case 1 | Baseline | Verify no false trip | Stable temperature and current policy |
| Case 2 | Thermal derating | Prove controlled current reduction | Current scale reduces; dT/dt falls |
| Case 3 | Runaway prevention | Prove early precursor and shutdown window | Warning, derate, then shutdown |
| Case 4 | Aging warning | Mild health-based derating | Current allowed around 0.7 |
| Case 5 | Severe aging | Strong health-based derating | Current allowed near 0.25 |
| Case 6 | Fast fault | Validate ms-level fault handling | Shutdown within 10 ms |

12.3 Case 3: Runaway Prevention

Case 3 is the most important thermal safety case. The precursor is generated long before final shutdown. The system creates a safety window in which it can derate and then isolate the battery if the hazard continues. This demonstrates anticipatory protection instead of late alarm behavior.

Figure 12.3: Case 3 runaway-prevention timeline and no-shutdown proxy comparison.

Table 12.2: Runaway-prevention phase durations.

| Phase | Duration | Meaning |
| --- | --- | --- |
| Precursor to derate | 1126.81 s | Long early-warning and monitoring interval |
| Derate to shutdown | 356.52 s | Intervention period before final isolation |
| Shutdown entry window | 50.00 s | Final shutdown activation window |
| Total safety window | 1483.33 s | Time between precursor and shutdown |

12.4 Comparative Result Dashboard

Figure 12.4: Comparative dashboard of protection windows, derating benefit, current policy, and fast-fault latency.

The dashboard summarizes the engineering contribution of the project. It shows that each protection case serves a different purpose: thermal derating reduces thermal drive, runaway prevention creates a safety window, aging policy reduces allowable current based on health, and fast fault protection operates in milliseconds.

12.5 What Each Feature Solves

Table 12.3: Feature-to-objective validation summary.

| Feature | Objective | Problem Solved | Advanced Method | Result / Significance |
| --- | --- | --- | --- | --- |
| Adaptive calibration | Improve measurement accuracy | Sensor offset and drift | REF/MEAS switching and correction hold | 78.6% RMS error reduction |
| Thermal derating | Avoid late-only shutdown | Rising heat during stress | dT/dt and stress-based scaling | 67% reduction in dT/dt |
| Runaway prevention | Create early safety window | Threshold systems react late | Precursor + derate + shutdown | 1483 s total safety window |
| Aging policy | Adapt to degradation | Fixed current ignores R0 rise | Health/R0/age model | Severe current scale near 0.251 |
| Fast fault | React to short-circuit quickly | Thermal path too slow | OC/UV fusion + RC hold | 7 ms shutdown |
| ADC/SAR | Digitize AFE signals | Digital BMS needs sampled data | S/H, quantization, SAR DAC | Mixed-signal readiness |
| RLS estimation | Track internal resistance | Pulse method is limited | Recursive Least Squares | Online R0 diagnostic signal |

Chapter 15

Engineering Significance and Future Scope

13.1 Engineering Significance

The strongest aspect of the project is not any single block but the integration of multiple safety and diagnostic layers. The front-end does not only sense signals; it validates them, calibrates them, digitizes them, estimates degradation, computes risk, and drives protection decisions. This makes the architecture closer to a real battery intelligence system.

13.2 Comparison with Simple Threshold Protection

Table 13.1: Comparison between conventional threshold BMS and proposed architecture.

| Conventional Threshold BMS | Proposed Architecture |
| --- | --- |
| Acts only after fixed voltage/current/temperature limit is crossed | Uses warning, derating, precursor, and shutdown stages |
| Usually ignores measurement offset | Includes reference-based adaptive calibration |
| Often uses fixed current limits | Adapts current using R0eff, age stress, risk score, and health score |
| Thermal and electrical faults may share slow protection path | Uses separate millisecond fast-fault path |
| Limited analytics after simulation | Uses Python validation with case metrics and policy fitting |
| May not include digital interface validation | Includes ADC/SAR monitoring support |

13.3 Limitations

The project is simulation-validated. It does not claim silicon implementation or hardware pack testing. Some blocks use behavioral sources to represent analog and mixed-signal behavior. The ADC and SAR blocks are functional mixed-signal demonstrations rather than transistor-level converter layouts. Thermal parameters are selected for scenario validation and would need tuning using real cell data for deployment.

13.4 Future Scope

• Replace behavioral comparators with transistor-level comparator and latch circuits.

• Implement the ADC/SAR subsystem using a full capacitor DAC and real SAR logic.

• Validate the model using real cell pulse-test data and thermal chamber measurements.

• Add cell balancing and multi-cell pack monitoring.

• Implement the supervisory logic on FPGA or microcontroller.

• Move the analog front-end toward ASIC-level design using Cadence or an equivalent EDA flow.

• Integrate SOC/SOH estimation using Kalman filtering or machine-learning-assisted parameter tracking.

• Add hardware-in-the-loop testing with real sensors and controlled battery load profiles.

13.5 Resume and Viva Positioning

This project can be positioned as a hierarchical EV battery safety and diagnostics platform. The core phrase is: self-calibrated mixed-signal battery front-end with predictive thermal safety, aging-aware current policy, and sub-10 ms fast-fault shutdown. In viva, the most important points to emphasize are the layered time-scale design, the reason for derating before shutdown, the use of internal resistance for aging, and the role of calibration before estimation.

Chapter 14: Conclusion

This project successfully demonstrates a simulation-validated intelligent analog front-end for EV battery diagnostics and protection. The system combines sensing, calibration, ADC support, electro-thermal modeling, RLS-based internal resistance estimation, aging-aware current governance, fast derating, runaway precursor detection, and fast fault shutdown. The work shows that battery protection should not be limited to simple threshold-based cutoff. A practical EV battery system must respond differently to slow thermal stress, long-term aging, and millisecond electrical faults.

Case-wise validation confirms that the baseline case remains stable, the thermal derating case reduces current and suppresses dT/dt, the runaway-prevention case creates a long warning and intervention window, the aging cases adjust current according to degradation, and the fast-fault case achieves approximately 7 ms shutdown against a 10 ms target. The calibration subsystem reduces measurement error, while ADC/SAR and RLS blocks make the architecture suitable for future mixed-signal and embedded BMS integration.

Overall, the project establishes a complete hierarchical battery intelligence framework. It is not only a thermal protection circuit but a multi-layer EV battery safety stack that combines analog front-end design with predictive diagnostics and quantitative validation.

REFERENCES

[1] M. Chen and G. A. Rincon-Mora, "Accurate electrical battery model capable of predicting runtime and I-V performance," IEEE Transactions on Energy Conversion, 2006.

[2] G. L. Plett, Battery Management Systems, Volume I: Battery Modeling. Artech House, 2015.

[3] G. L. Plett, Battery Management Systems, Volume II: Equivalent-Circuit Methods. Artech House, 2015.

[4] D. Linden and T. B. Reddy, Handbook of Batteries. McGraw-Hill, 2011.

[5] R. Xiong, Battery Management Algorithm for Electric Vehicles. Springer, 2020.

[6] LTspice 26.0.1 documentation and simulation environment, Analog Devices.

[7] S. Haykin, Adaptive Filter Theory. Pearson, 2014.

[8] B. Razavi, Design of Analog CMOS Integrated Circuits. McGraw-Hill, 2001.

[9] R. Jacob Baker, CMOS: Circuit Design, Layout, and Simulation. Wiley-IEEE Press, 2010.

[10] A. Jossen, "Fundamentals of battery dynamics," Journal of Power Sources, 2006.

Appendix A: Detailed Feature-to-Objective Mapping

This appendix summarizes how every major subsystem connects to a specific project objective. The purpose is to make clear that the system is not a collection of disconnected plots; every block exists to solve a defined engineering problem.

Table A.1: Detailed feature-to-objective mapping.

| Subsystem | Objective Solved | Approach Used | Mentor-Level Significance |
| --- | --- | --- | --- |
| Voltage/current sensing | Acquire battery electrical response | ECM, shunt current, filtered sense nodes | Creates the analog basis for all protection and estimation decisions. |
| Adaptive calibration | Remove measurement drift/offset | Reference switching, error capture, correction hold | Improves trust in sensed signals before estimation or protection. |
| Electro-thermal model | Predict heating behavior | Core-surface thermal RC network with Qirr, Qrev, Qrxn | Captures slow thermal stress and internal heat acceleration. |
| Thermal derating | Avoid late shutdown | Stress-driven current scaling and charge limit | Shows controlled intervention instead of abrupt cutoff. |
| Runaway prevention | Detect early runaway signature | Precursor logic using dT/dt, Tcell and gradient | Provides long warning and intervention windows. |
| Aging policy | Adapt to degraded cell condition | R0eff, age stress, health score, current scale | Avoids unsafe fixed-current behavior for aged batteries. |
| Fast fault logic | Protect against short-circuit/over-current | Comparator fusion, RC validation, hold latch | Adds millisecond protection path independent of thermal response. |
| ADC/SAR layer | Prepare analog signals for digital BMS | S/H, quantization, bit extraction and SAR DAC | Makes the front-end mixed-signal and controller-ready. |
| RLS estimation | Estimate internal resistance online | Recursive Least Squares using voltage-current response | Connects diagnostics with aging-aware control policy. |

Appendix B: Algorithmic Flow Used in the Protection Stack

B.1 Calibration Algorithm

Step 1: In measurement mode, route the battery sense voltage to the measurement path.

Step 2: In reference mode, route the known calibration reference into the same path.

Step 3: Compute raw error between reference and observed output.

Step 4: Store the correction on the hold path.

Step 5: Apply the correction during measurement mode.

Step 6: Use validity flags to decide whether the calibrated output is trusted.

B.2 Thermal Derating Algorithm

Step 1: Compute Tcell_abs, thermal gradient and filtered dT/dt.

Step 2: Evaluate warning and critical thresholds.

Step 3: Generate CHG_LIMIT when thermal stress crosses the derating region.

Step 4: Reduce ICMD_SCALE_FINAL using charge-limit and stress scaling.

Step 5: If critical thermal conditions persist, generate THERM_SHDN and FINAL_SHDN.

B.3 Aging Policy Algorithm

Step 1: Compute R0eff using the battery health model.

Step 2: Convert R0eff into AGE_STRESS.

Step 3: Generate AGING_WARN or AGING_SEVERE.

Step 4: Map the aging state to AGE_SCALE.

Step 5: Combine AGE_SCALE with thermal and trust limits to compute final allowable current. This prevents an aged cell from being charged as aggressively as a healthy cell.

B.4 Fast Fault Algorithm

Step 1: Compare current with fast over-current threshold.

Step 2: Compare voltage with fast under-voltage threshold.

Step 3: Fuse OC and UV information to detect short-circuit-like behavior.

Step 4: Pass the raw fault through an RC validation network.

Step 5: Store the fault through a hold node.

Step 6: Assert FAST_SHDN and combine it with thermal shutdown to create FINAL_SHDN.

• Fast layer response: milliseconds.

• Thermal derating response: seconds to minutes.

• Runaway precursor response: early warning and intervention window.

Appendix C: Case-by-Case Interpretation for Viva

C.1 Case 1 - Baseline

Case 1 proves that the architecture is not over-sensitive. The cell remains in a healthy operating region, thermal stress is low, and no shutdown event is produced. This is important because a protection system that trips during normal operation is not useful.

C.2 Case 2 - Thermal Derating

Case 2 is used to demonstrate controlled intervention. The current is reduced before thermal shutdown, and the dT/dt response decreases. The key defense point is that derating is a softer and more practical response than immediate shutdown.

C.3 Case 3 - Runaway Prevention

Case 3 is the strongest thermal safety case. It shows precursor detection, derating, and final shutdown. The long warning window demonstrates that the system has time to act before a severe event. This answers the mentor question: what is special compared with a normal temperature cutoff?

C.4 Case 4 and Case 5 - Aging Policy

Cases 4 and 5 prove that the system adapts with battery health. Case 4 has mild aging and therefore allows more current. Case 5 has severe aging and sharply restricts current. This is important because internal resistance directly affects heat generation.

C.5 Case 6 - Fast Fault

Case 6 validates that electrical faults are handled separately from thermal faults. A short-circuit cannot wait for cell temperature to rise. The observed shutdown latency of approximately 7 ms satisfies the 10 ms target and proves that the fast path is effective.

Figure C.1: Detailed thermal derating waveform used for Case 2 defense.

Figure C.2: Detailed runaway prevention waveform used for Case 3 defense.

Appendix D: Key LTspice Behavioral Blocks

The LTspice implementation uses behavioral sources to model system-level analog and mixed-signal behavior. These blocks make the design compact enough for simulation while still representing physical and logical relationships. The following descriptions summarize the key behavioral blocks used in the design.

Table D.1: Key LTspice behavioral blocks.

| LTspice Block | Purpose | Explanation |
| --- | --- | --- |
| B_R0EFF | Effective internal resistance | Models resistance variation with temperature, SOC and aging. |
| B_QIRR | Irreversible heat | Computes I^2R heat, the main heat source during high current. |
| B_QREV | Reversible heat | Models entropic contribution based on current, temperature and SOC. |
| B_QRXN | Reaction heat | Represents accelerated heat generation near runaway conditions. |
| B_PRECURSOR | Runaway precursor | Combines warning signals to detect early runaway tendency. |
| B_CHG_LIMIT | Charge limit | Triggers derating when thermal warnings accumulate. |
| B_ICMD_SCALE_FINAL | Final current scale | Combines thermal, aging and measurement-trust constraints. |
| B_FAST_FAULT_RAW | Fast electrical fault | Fuses over-current and short-circuit evidence. |
| B_FINAL_SHDN | Final shutdown | OR-combines thermal shutdown and fast shutdown. |
| B_MEAS_TRUST_OK | Measurement trust | Blocks unsafe operation if sensor path is invalid. |

The design deliberately separates diagnostic variables from action variables. For example, R0eff and THERM_STRESS are diagnostic indicators, while ICMD_SCALE_FINAL and FINAL_SHDN are action outputs. This separation helps explain the project clearly during evaluation.

D.1 Representative Equations

Q_gen = Q_irr + Q_rev + Q_rxn

Q_irr = I^2 (R0_eff + eta R1heat)

THERM_STRESS = limit((Tcell_abs - 35) / 20, 0, 1)

AGE_STRESS = limit((R0_eff - 0.06) / 0.10, 0, 1)

RISK_SCORE = 0.6 THERM_STRESS + 0.4 AGE_STRESS

FINAL_SHDN = FAST_SHDN OR THERM_SHDN_F

Appendix E: Selected Figures for Final Demonstration

The following figures are the most useful for presentation and mentor discussion. They should be prioritized in the final PPT or project viva because they directly support the novelty claims of the project.

Figure E.1: Calibration timing and measurement/reference switching.

Figure E.2: Calibration error reduction summary.

Figure E.3: Aging policy model validation.

Figure E.4: RLS-based online internal resistance estimate.